CFD Packed Column Models for Points Source Carbon Capture

Yucheng Fu1, Rajesh Kumar Singh1, Chao Zeng1, Jie Bao1, Zhijie Xu1

1Pacific Northwest National Laboratory, Richland, WA 99352, USA

This document outlines the CFD models developed to analyze the performance of packed columns in point source carbon capture applications. The models incorporate three distinct packing types: random packing[1–4], structure packing[5–8] and triply periodic minimal surface (TPMS) packing [9].  These CFD models are available in both Stereolithography (.stl) and Nastran (.nas) file formats.

# Random Packing

## Raschig ring representative column

Packing Element: Raschig ring

Column size: H = 6 cm, ID = 7.62 cm

Model description: random packed column filled Raschig ring elements. The specific area of the column is ap = 857.5 m2/m3

Figure 1 Visualization of (a) single element and, (b) Raschig ring representative column CFD model.

## Pro-Pak representative column

Packing element: Pro-Pak

Column size: H = 6 cm, ID = 7.62 cm

Model description: random packed column filled Pro-Pak elements. The specific area of the column is ap = 939.5 m2/m3

Figure 2 Visualization of (a) single element and (b) Pro-Pak ring representative column CFD model.

## Pall ring representative column

Packing element: Pall ring

Column size: H = 6 cm, ID = 10 cm

Model description: random packed column filled Pall ring elements. The specific area of the column is ap = 209.1 m2/m3

Figure 3 Visualization of (a) single element and (b) Pall ring representative column CFD model.

## Raschig ring bench-scale column

Packing element: Raschig ring

Column size: H = 53.3 cm, ID = 7.62 cm

Model description: random packed column filled with over eight thousand Raschig ring elements. The model is a full-size bench-scale column with a packing height over 50 cm. The solvent can be injected using single or multiple inlets setup to investigate the solvent distribution process during the operation.

Figure 4 Visualization of Raschig ring bench-scale column CFD model.

# Structure Packing

## M250Y periodic column

Packing element: Mellapak 250Y

Column size: Height = 10 cm, Width = 9.8 cm, Depth = 8 cm

Model description: Structure packed column filled with Mellapak 250Y sheets. The periodic domain contains 3 x 3 x 3 repeated units of the Mellapak 250Y in the computational domain.

Figure 5 Visualization of Mellapak 250Y periodic column CFD model.

## M500Y representative column

Packing element: Mellapak 500Y

Column size: H = 20 cm, ID = 7.62 cm

Model description: Structure packed column filled Mellapak 500Y sheets. Two packing units are stacked in the column with a rotation of 90 degree for the second unit.

Figure 6 Visualization of Mellapak 500Y representative column CFD model.

# TPMS

## Schwarz-D representative column

Packing: Schwarz-D surface

Column size: H = 7 cm, ID = 7.62 cm

Model description: packed column filled Schwarz-D TPMS surface with an average packing wall thickness around 0.4 mm. The specific area of the column is ap = 620 m2/m3

Figure 7 Visualization of Schwarz-D representative column CFD model.

## Gyroid representative column

Packing: Gyroid surface

Column size: H = 7 cm, ID = 7.62 cm

Model description: packed column filled Gyroid TPMS surface with an average surface wall thickness around 0.2 mm. The specific area of the column is ap = 607 m2/m3

Figure 8 Visualization of Gyroid representative column CFD model.

# Reference

[1]	Y. Fu, J. Bao, R. Singh, C. Wang, Z. Xu, Investigation of countercurrent flow profile and liquid holdup in random packed column with local CFD data, Chem Eng Sci. 221 (2020). https://doi.org/10.1016/j.ces.2020.115693.

[2]	B. Bartoldson, R. Wang, Y. Fu, D. Widemann, S. Nguyen, J. Bao, Z. Xu, B. Ng, Latent Space Simulation for Carbon Capture Design Optimization, ArXiv. (2021).

[3]	B. Bartoldson, R. Wang, Y. Fu, D. Widemann, S. Nguyen, J. Bao, Z. Xu, B. Ng, Latent Space Simulation for Carbon Capture Design Optimization, in: Proceedings of the 36th AAAI Conference on Artificial Intelligence, AAAI 2022, 2022.

[4]	Y. Fu, J. Bao, R.K. Singh, R.F. Zheng, C.M. Anderson-Cook, K.S. Bhat, Z. Xu, The influence of random packed column parameters on the liquid holdup and interfacial area, AIChE Journal. 68 (2022). https://doi.org/10.1002/aic.17691.

[5]	R.K. Singh, J. Bao, C. Wang, Y. Fu, Z. Xu, Hydrodynamics of countercurrent flows in a structured packed column: Effects of initial wetting and dynamic contact angle, Chemical Engineering Journal. 398 (2020). https://doi.org/10.1016/j.cej.2020.125548.

[6]	R.K. Singh, J.E. Galvin, X. Sun, Hydrodynamics of the rivulet flow over corrugated sheet used in structured packings, International Journal of Greenhouse Gas Control. 64 (2017) 87–98. https://doi.org/10.1016/j.ijggc.2017.07.005.

[7]	R.K. Singh, J.E. Galvin, X. Sun, Multiphase flow studies for microscale hydrodynamics in the structured packed column, Chemical Engineering Journal. 353 (2018) 949–963. https://doi.org/10.1016/j.cej.2018.07.067.

[8]	C. Zeng, Y. Fu, R. Singh, J. Bao, Z. Xu, Differentiation of static and dynamic interfacial area in the structured packed column, Chem Eng Sci. 260 (2022). https://doi.org/10.1016/j.ces.2022.117877.

[9]	R.K. Singh, Y. Fu, C. Zeng, D.T. Nguyen, P. Roy, J. Bao, Z. Xu, G. Panagakos, Hydrodynamics of countercurrent flow in an additive-manufactured column with triply periodic minimal surfaces for carbon dioxide capture, Chemical Engineering Journal. 450 (2022) 138124. https://doi.org/10.1016/j.cej.2022.138124.